# DevLab PDM ICS-41350 MEMS Microphone

Product Reference — Hardware V1.1 — Mfr. Part #: UE0148

UNIT Electronics

2026-08-03

## Contents

- Description

  - Applications

  - Hardware Features

- 1 The Module

  - 1.1 Board Identification

  - 1.2 Package Contents

- 2 Electrical Characteristics

  - 2.1 ICS-41350 Characteristics

  - 2.2 Electrical Precautions

- 3 Functional Overview

  - 3.1 Audio Signal Path

  - 3.2 Operating Modes

  - 3.3 Channel Selection

  - 3.4 Power Section

- 4 Connectors and Pinout

  - 4.1 Edge Connections

  - 4.2 Four-Position Connector

  - 4.3 Solder Options

- 5 Board Operation

- 6 Mechanical Information

- 7 Company Information

- 8 Reference Documentation

- 9 Appendix

  - 9.1 Source Hierarchy

  - 9.2 Items Pending Validation

  - 9.3 Document Control

## Description

The UNIT DevLab PDM ICS-41350 MEMS Microphone is a compact digital-audio module built around the bottom-port TDK InvenSense ICS-41350. The microphone contains a MEMS element, impedance-converter amplifier, and fourth-order sigma-delta modulator that produces a 1-bit pulse-density-modulated output.

### Applications

- Voice capture and recognition prototypes

- Microphone-array development

- Camera, security, and surveillance audio

- Low-power ambient sound analysis and keyword spotting

- Ultrasonic sensing experiments in high-performance mode

### Hardware Features

- ICS-41350 bottom-port digital MEMS microphone

- PDM clock and data interface

- Clock-selected low-power, standard, high-performance, and sleep modes

- Sensor channel selection for shared-data stereo configurations

- AP2112K-3.3TRG1 fixed 3.3 V regulator identified by the BOM

- Six 2.54 mm edge/header connections

- Four-position, 1.0 mm-pitch right-angle connector

- Orange indicator LED and two solder options shown on the V1.1 artwork

## 1 The Module

### 1.1 Board Identification

| Item | Value |
| --- | --- |
| Product | DevLab PDM ICS-41350 MEMS Microphone |
| Manufacturer part number | UE0148 |
| Hardware revision shown | V1.1 |
| Microphone | TDK InvenSense ICS-41350 |
| Digital audio interface | PDM |
| Product-reference revision | 0.1.0 |

### 1.2 Package Contents

The manufacturing BOM includes one four-conductor, 1.0 mm-pitch QWIIC-style harness. Confirm purchased-product contents with the distributor. The BOM identifies J2 as a four-position, 1.0 mm-pitch connector, but cable orientation and end-to-end pin order remain pending validation.

## 2 Electrical Characteristics

### 2.1 ICS-41350 Characteristics

The values in this section apply to the ICS-41350 sensor. They are not complete module ratings.

| Parameter | Low-power mode | Standard mode | High-performance mode | Scope |
| --- | --- | --- | --- | --- |
| Clock frequency | 400–800 kHz | 1.0–3.3 MHz | 4.1–4.8 MHz | Sensor CLK |
| Sensitivity, typical | −26 dB FS | −26 dB FS | −32 dB FS | Sensor, 1 kHz and 94 dB SPL |
| SNR, typical | 63 dBA | 64 dBA | 64 dBA | Sensor |
| Supply current, typical | 185 µA | 430 µA | 650 µA | Sensor at VDD = 1.8 V, no load |
| Acoustic overload point, typical | 120 dB SPL | 120 dB SPL | 126 dB SPL | Sensor, 10% THD |

| Parameter | Minimum | Typical | Maximum | Scope |
| --- | --- | --- | --- | --- |
| Sensor supply voltage, VDD | 1.65 V | — | 3.63 V | ICS-41350 supply pin |
| Sleep-mode clock | — | — | Below 200 kHz | ICS-41350 CLK |
| Sleep current | — | 12 µA | 20 µA | ICS-41350 |
| Digital input high, VIH | 0.65 × VDD | — | — | ICS-41350 digital input |
| Digital input low, VIL | — | — | 0.35 × VDD | ICS-41350 digital input |
| Regulated output | — | 3.3 V nominal | — | U3 identity from BOM; module behavior pending |

The permitted voltage at module VIN, the behavior and limits of VSYS, host logic compatibility, and total module current are pending schematic verification and module-level validation. Do not substitute the sensor VDD range, the regulator limits, the diode ratings, or the connector rating for a module input rating.

### 2.2 Electrical Precautions

- Keep the host and module grounds connected during operation.

- Do not drive DATA; it is the microphone output.

- Apply CLK and CH only after the module power and logic domains have been verified against a released schematic or a measured board.

- Keep the bottom acoustic port free from debris, flux, adhesive, and enclosure obstruction.

- Verify VIN, VSYS, J2 supply routing, and solder-option states before first power-up.

## 3 Functional Overview

### 3.1 Audio Signal Path

The ICS-41350 senses pressure through its bottom acoustic port and emits the 1-bit output of a fourth-order sigma-delta modulator on DATA. A host supplies CLK and performs PDM decimation or passes the stream to a compatible codec.

### 3.2 Operating Modes

The sensor selects its operating mode from the applied clock frequency: below 200 kHz for sleep, 400 to 800 kHz for low power, 1.0 to 3.3 MHz for standard, and 4.1 to 4.8 MHz for high performance. These are ICS-41350 characteristics; tested module and host configurations remain pending validation.

### 3.3 Channel Selection

At the ICS-41350 SELECT pin, low selects the right channel (DATA1) and high, tied to sensor VDD, selects the left channel (DATA2). The V1.1 board artwork labels an exposed CH connection and a CH SELECTION L/R solder option. Their exact circuit mapping and default state are pending schematic verification.

### 3.4 Power Section

The BOM identifies an AP2112K-3.3TRG1 regulator, NSR0320MW2T1G Schottky diode, 1 µF power capacitors, a 100 nF microphone decoupling capacitor, an orange indicator LED, and associated resistors. The exact VIN/VSYS path and the LED-enable default remain pending schematic verification.

## 4 Connectors and Pinout

### 4.1 Edge Connections

| Board label | Direction | Function |
| --- | --- | --- |
| GND | Power | Common ground |
| VIN | Input | Module power-section input; allowed range pending |
| VSYS | Power | Module system rail; electrical behavior pending |
| CLK | Input | PDM clock input |
| DATA | Output | PDM microphone data |
| CH | Input | Channel-selection signal; module mapping pending schematic verification |

### 4.2 Four-Position Connector

The V1.1 top artwork labels the connector signals, from top to bottom in the rendered view, as CLK, DATA, VSYS, and GND. The manufacturing BOM calls J2 a four-position, 1.0 mm-pitch right-angle connector. Verify physical orientation before connecting a cable; this edition does not assign connector pin numbers.

### 4.3 Solder Options

The bottom artwork identifies CH SELECTION L/R and LED ON ENABLE options. Their default copper states and exact circuit connections are pending schematic verification.

## 5 Board Operation

No firmware example has been validated for this module, so this reference does not present host code as a tested procedure.

For initial bench validation:

- Inspect the board and ensure the bottom acoustic port is unobstructed.

- Verify the VIN/VSYS power path, supply range, and logic domain against a released schematic or measured board.

- Connect host and module grounds.

- Connect the host PDM clock output to CLK and PDM data input to DATA.

- Configure the channel only after verifying how board CH and the solder option connect to the ICS-41350 SELECT pin.

- Apply a clock in the intended ICS-41350 mode range and verify timing with a logic analyzer before relying on captured samples.

Host GPIO numbers are controller-specific and are not module pin numbers. Tested decimation settings, sample rates, host mappings, and module-level logic limits remain pending validation.

## 6 Mechanical Information

The rendered V1.1 artwork shows six castellated edge connections, a four-position right-angle connector, and a central circular opening. A controlled dimensional drawing and mounting coordinates are not available, so dimensions must not be scaled from the render.

The ICS-41350 itself uses a 3.5 mm × 2.65 mm × 0.98 mm bottom-port package. Those sensor-package dimensions are not the dimensions of the module. Allow clearance around the acoustic path and keep it free of adhesive, debris, and mounting obstruction.

## 7 Company Information

| Item | Information |
| --- | --- |
| Company | UNIT Electronics |
| Website | https://uelectronics.com/ |
| Address | Salvador 19, Cuauhtémoc, 06000 Mexico City, CDMX, Mexico |

Product specifications and documentation may be revised. Use the repository and product page listed in the next chapter for the latest released material.

## 8 Reference Documentation

| Reference | Location |
| --- | --- |
| Repository | Public URL pending |
| Hardware notes | Public URL pending |
| Microphone datasheet | TDK InvenSense ICS-41350 |
| Manufacturing BOM | Reviewed local source; public URL pending |

The local microphone source is TDK InvenSense document DS-000047, revision 1.1, dated 2016-06-23. Sensor values in this reference come from that document; module component identities come from the manufacturing BOM; visible module labels come from the V1.1 artwork.

## 9 Appendix

### 9.1 Source Hierarchy

This product reference follows the V1.1 board artwork for visible signal labels, the manufacturing BOM for fitted component identities, and TDK InvenSense datasheet DS-000047 revision 1.1 for ICS-41350 characteristics. A released module schematic and module-level validation take precedence if they conflict with this edition.

### 9.2 Items Pending Validation

- Module schematic and controlled connector pin-number drawing

- Module VIN and VSYS operating and absolute-maximum ratings

- Module logic levels and total current consumption

- Tested PDM clock modes, decimation settings, sample rates, and host mappings

- Board dimensions and mounting coordinates

- Solder-option defaults and J2 cable orientation

- Module-level acoustic, environmental, and ultrasonic characterization

### 9.3 Document Control

| Field | Value |
| --- | --- |
| Product | DevLab PDM ICS-41350 MEMS Microphone |
| Manufacturer part number | UE0148 |
| Hardware artwork covered | V1.1 |
| Product-reference revision | 0.1.0 |
| Publication date | 2026-08-03 |
| Firmware status | No validated example included in this edition |